The above features are enabled by the next conditions: if the speed is set and if the configuration is set.

Boot Lid

[DRL001-A-v1] Pressing the outer boot lid release button 1, if the car is driver door unlocked or centrally unlocked, shall activate the boot lid release motor.

[DRL002-A-v2] Pressing the outer boot lid release button 2, shall independent of the locking status, activate the boot lid release motor [if the car has an enabled KVM]. The PJB shall follow the MS CAN signal “KVM_Passive_Key_Request = 5 (TrunkUnlockReq)” within message #215 (KVM_Request).

[DRL003-B-v3] Pressing the boot lid release button on the RC option 2 (default) twice within 3 seconds, shall independent of the locking status, activate the boot lid release motor [if the car is not configured for boot lid release with one RC button press].

[RHT_PJB_DRL007-A-v4] If the PJB is configured for S389 (S389_Conf=enabled) and receives a valid boot lid release request from the CRM (CAN signal CRM_TaillatchOpeningReq = open and change of the CRM_TaillatchOpeningCnt), the boot lid release motor shall be activated by CRM request independent of the vehicle speed.

[DRL007-A-v1] If the car is centrally unlocked, the boot lid release motor shall be activated by the boot lid remote control release button, if the boot lid release button is pressed within “T_EnableBootLidRelease” seconds after the remote control boot lid release button was pressed once.

[DRL007-A-v1] If the car is double locked, the boot lid release motor shall be activated by the boot lid remote control release button, if the boot lid release button is pressed within “T_EnableBootLidRelease” seconds after the remote control boot lid release button was pressed once.

Mini lift gate

[MLRL002-A-v1] Pressing the mini lift gate release by outer release button 1, if the car is driver door unlocked or centrally unlocked shall activate the mini lift gate release motor.

[MLRL003-A-v1] Pressing the Mini lift gate release by outer release button 2 shall independent of the locking status activate the mini lift gate release motor [if the car has an enabled KVM]. The PJB shall follow the MS CAN signal “KVM_Passive_Key_Request = 6 (BackliteReleaseReq)” within message #215 (KVM_Request).

[MLRL004-A-v1] If the car is centrally locked or double locked, the mini lift gate release motor shall be activated by the outer release button 3, if the mini lift gate release is pressed within “T_EnableBootLidRelease” seconds after the remote control mini lift gate (boot lid) release button was pressed once.

[MLRL004-A-v1] If the configuration parameter “T_EnableBootLidRelease = 0” is set, the functionality shall be disabled by the outer release button 3.

[MLRL005-A-v1] Pressing the remote control boot lid release button on the RC once for “T_tailgate_longpress_timer” shall independent of the locking status activate the mini lift gate release motor.

Speed

[DRL004-A-v1] the boot lid release feature shall be enabled by any boot lid release button, if the ignition is run or start and the vehicle speed is no more than 7 km/h.

[DRL004-A-v1] the boot lid release feature shall be enabled by any boot lid release button, if the ignition is off, independent of the actual vehicle speed information.

[DRL004-A-v1] the boot lid release feature shall be enabled by any boot lid release button, if the vehicle speed fault bit is set, independent of the actual vehicle speed information.

Configuration

[DRL008-A-v1] The boot lid release feature shall be activated by the configuration, if the vehicle is configured for mini-lift gate (MINI-LIFT GATE is different of 1) and the mini-lift gate is closed (PJB_RearWindowStatus = 0 (rear window ajar) within CAN message 433).

[MLRL008-A-v1] The mini lift gate release feature shall be activated by the configuration, if the boot lid is closed (PJB_BootLidStatus different of 0 (trunk ajar) within the CAN message 433).

[DRL008-A-v1] The boot lid release feature shall be activated by the configuration, if the CAN message is not missing.

[DRL008-A-v1] The boot lid release feature shall be activated by the configuration, if the configuration parameter T_EnableBootLidRelease different of 0 is set.

[MLRL001-A-v1] The configuration parameter MINI_LIFTGATE_CONF = 1 shall be set by the configuration.

Exceptions

[RHT_PJB_DRL008-A-v1] If the PJB has detected a “missing CAN signal” on the signals CRM_TaillatchOpeningReq and CRM_TaillatchOpeningCnt (CAN message 0x230 missing) or after a Power On Reset (POR) the first messages after (again) receiving the message shall be used for re-synchronisation.

[RHT_PJB_DRL008-A-v1] Only the next change of the signal counter (CRM_TaillatchOpeningCnt++) together with an opening request signal (CRM_TaillatchOpening = open) shall be accepted as a new CRM request.

[DRL006-A-v1] The boot lid release by outer release switch will be enabled by any boot lid release button, if after a power on reset (Lock Status = unknown) one door with key-cylinder is unlocked.

[MLRL007-A-v1] The mini lift gate release by outer release switch will be enabled by any boot lid release button, if after a power on reset (Lock Status = unknown) one door with key-cylinder is unlocked.